|  | Lead-Acid Battery (Car) | Nickel Metal-Hydride Battery (AA Cell) | Lithium-Ion Battery (Mobile Phone) |
| --- | --- | --- | --- |
| Voltage | 12 V | 1.2 V | 3.7 V |
| Mass | 25 kg | 30 g | 20 g |
| Capacity | 70 Ah | 2000 mAh |  |
| Energy Density |  |  | 185 Wh/kg |

|  | Lead-Acid Battery (Car) | Nickel Metal-Hydride Battery (AA Cell) | Lithium-Ion Battery (Mobile Phone) |
| --- | --- | --- | --- |
| Voltage | 12 V | 1.2 V | 3.7 V |
| Mass | 25 kg | 30 g | 20 g |
| Capacity | 70 Ah | 2000 mAh | (x ∙ 3.7 V) / 0.02 kg = 185 Wh/kg  x = 1 Ah = 1000 mAh |
| Energy Density | (70 Ah ∙ 12 V) / 25 kg = 33.6 Wh/kg | (2 Ah ∙ 1.2 V) / 0.03 kg = 80 Wh/kg | 185 Wh/kg |