Etching a Printed Circuit Board

A printed circuit board, or PCB, is used to mechanically support and electrically connect electronic components using conductive pathways, tracks or signal traces etched from copper sheets laminated onto a non-conductive substrate. Printed circuit boards are used in virtually all but the simplest commercially produced electronic devices.

Conducting layers are typically made of thin copper foil (35 µm) on plastics.

Fig. 1: 	The heart of a computer: 
a motherboard

Fig. 2: 	A PCB that has been etched but not yet populated with electronic components

On the copper layer a coating is applied that can be hardened by UV light. A photomask is used so that the coating is only hardened where the copper layer shall be protected. The remaining coating protects the copper foil. Subsequent etching removes the unwanted copper. The result is shown in figure 2.

Task

- Draw a profile of the PCB and show what happens during etching.

- For industrial processes ferric chloride is often used as etchant. What do the two terms ”etchant“
and “ferric chloride“ mean?

”etchant“  = 		“ferric chloride“  =

- Write down the resulting redox equations:

- For us the above reaction is to slow. Therefore we use a mixture of hydrogen peroxide (H2O2) and hydrochloric acid.

Write down the resulting redox equations: